QTY 2 : Rugged Rack-Mounted Server Computer

Technical Specifications

Height shall be 3U or less

Length shall be less than 22”

Unit shall be air-cooled

QTY	Component

1 	REDUNDANT 1200WAC POWER SUPPLY, 120/240VAC INPUT

1 	MOTHERBOARD, SINGLE LGA6096, AMD EPYC™ 9004/9005 PROCESSORS, ATX, SUPERMICRO H14SSL-NT

1 	PROCESSOR, LGA6096 SP5, 3.2/4.4 GHZ, 64 CORE, 6400 MHZ DDR5, 360W, AMD EPYC 9555

8 	64GB MEMORY DIMM, RDIMM, DDR5, 6400MHZ

8 	SSD, NVME U.2, 12.8TB, TCG OPAL, FIPS 140-3

1 	SSD, SATA, 1.92TB, TCG-OPAL SED

1 	CARD, CONTROLLER, HBA, NVME, PCIE X16 GEN 5, LP, 2 PORT MCIO X8, PCI-E REDRIVER, MICRO SATA CABLES MCIO-2203-16X,

2 	EDV-01405, CARD, NETWORK, 2 PORT, 100GB, QSFP, PCI-E 3.0/4.0 X16, LP, INTEL E810-CQDA2,

1 	CARD, VIDEO, NVIDIA RTX PRO 6000 BLACKWELL MAX-Q WORKSTATION EDITION, 96GB GDDR7, 24064 CUDA CORES, 752 TENSOR

1 	MODULE, TPM 2.0, FIPS 140-2, HORIZONTAL, FOR SUPERMICRO X13/H13 AND X14/H14 BOARDS WITH 10 PIN HEADER, SUPERMICRO,

Environmental Requirements

MIL-STD-810, Operational Temperature, high	Procedure II: +50°C, two-hour dwell, four cycles

MIL-STD-810, Operational Temperature, low	Procedure II: -30°C, two-hour dwell, four cycles

MIL-STD-810, Altitude, Method 500	12,500 ft. operation, 40,000 ft. transport

MIL-STD-810, Thermal Shock, Method 503	Procedure II: 10 cycles, -40°C to +55°C, 15-min dwell, <1-min transfer time

MIL-STD-810, Humidity, Method 507	Procedure II: 240 hours with optional conformal coating kit

MIL-STD-810, Fungus, Method 508	28 days, mixed spore, 30°C 95% RH

MIL-STD-810, Salt fog, Method 509	48-hour test

MIL-STD-810, Sand-Dust, Method 510	Procedure I: Blasting dust, 12 hours

MIL-STD-810, Acceleration, Method 513	Procedure II: 9g

MIL-STD-810, Vibration, Method 514	Procedure I: 4.7G, 5–2,000Hz, 60 min/axis, 3 axis

MIL-STD-810, Shock, Method 516	Procedures I & V: 40G, 11ms, 18 pulses, 3/axis both directions

MIL-STD-1474E, Acoustic Noise	Requirement S, Grade A3

MIL-STD-167-1A, Ship Vibration	Type 1

MIL-S-901E, Shipboard Shock	Class II, A/B

Electromagnetic Compatibility Requirements

MIL-STD-461	EMI/EMC, RE102, CE102; surface ship, below deck, and ground

RTCA/DO-160	Aircraft and airborne equipment, Category M