STATEMENT OF WORK

Wash Rack Service Agreement

- Background

  - The Property and Vehicle Maintenance Group of the Hoover Dam, Boulder City, NV requires the current wash rack electric pressure washer and water recycle system to be maintained to factory specifications on a scheduled basis.

  - The wash rack pressure washer is used daily to clean vehicles and equipment and is integral to maintenance and care of these assets.

- Scope

  - The government expects to have the LP Gas fired, electric hot water pressure washer Service and Maintained.

  - The government expects to have the water recycle system Serviced and Maintained.

- Objectives

  - The result is to have a maintenance plan in place to ensure longevity and efficient, properly performing Recycle System.

  - The maintenance plan shall be conducted on a regular basis at an interval of no less than once per month.

- Tasks

  - This is a Detailed Statement of Work

  - The tasks the contractor is to complete are as follows:

    - Thoroughly test and inspect complete Recycle System including peripheral equipment

      - Inspect any specific faults noted by Warehouse Personnel

    - Perform any routine maintenance

    - Test and inspect Reagent pumps

      - Check for air bubbles in lines

      - Adjust pumps settings as needed

    - Test and inspect skimming system

      - Inspect tank and scum level

      - Inspect skimming pipes

      - Adjust skim timer as needed

    - Test and inspect sludge removal system

      - Inspect sludge levels in coagulant and separator tanks

      - Inspect sludge socks for efficiency

      - Adjust sludge timer as needed

    - Check and calibrate pH sensor

      - Clean sensor bulb before calibration occurs

      - If necessary, mix buffer solution to maintain proper pH levels

    - Inspect and clean collection pit

      - When cleaning out the collection pit, remove all heavier solids and floatable oils and greases from wastewater.

    - Test and inspect all electrically energized components to include but not limited to:

      - Power switches

      - Breakers

      - Automatic flow control valves

      - Pumps

      - Wiring

      - Motors

      - Float switches

      - Etc..

    - Inspect all filter media

      - Cleaning filters and media if possible

      - Advise on replacement when necessary

- Replacement parts

  - If at any time a part fails or needs to be replaced, the contractor must inform the on-site maintenance mechanic before any work is done.

- Government Furnished Property:

  - None

- Security:

  - The contractor and its employees will be escorted by a Bureau of Reclamation employee in secure areas required to complete all tasks.

  - No contract employees will receive a badge.

- Place of Performance:

  - The contractor shall complete all items and perform all tasks at the Hoover Dam Central Warehouse:  U.S. Highway 93 / NV State Route 172, Boulder City, NV 89005

- Period of Performance