STATEMENT OF WORK (SOW)

Project Title: Roof Rehabilitation – TPO Recover Systems

Agency: U.S. Department of Agriculture (USDA), Forest Service

Forest: Lincoln National Forest

Location: Ruidoso, New Mexico

1.0 PROJECT OBJECTIVE

The objective of this project is to rehabilitate existing roofing systems at USDA Forest Service facilities by installing Thermoplastic Polyolefin (TPO) roof recover systems over existing metal and flat (tar/built-up) roofs, along with full replacement of gutters and downspouts.

The work shall extend roof service life, improve watertight integrity, and integrate with existing building systems, two roofs have current solar panels established, while minimizing disruption to Forest Service operations.

The Contractor shall furnish all labor, materials, equipment, supervision, and incidentals necessary to complete the work.

2.0 SCOPE OF WORK

2.1 GENERAL REQUIREMENTS

A. This project is a roof recover installation. Existing roofing systems shall remain in place except where localized removal is required to correct unsound conditions.

B. The Contractor shall verify that existing roof systems are structurally sound and suitable for installation of a TPO recover system prior to installation.

C. All work shall comply with:

TPO manufacturer requirements

Applicable ASTM standards

USDA Forest Service construction standards

2.2 EXISTING ROOF PREPARATION AND REPAIRS

A. Metal Roofs

The Contractor shall:

Inspect metal roofs for corrosion, loose panels, failed fasteners, damaged seams, and penetrations.

Remove and replace severely rusted or deteriorated metal panels where necessary.

Remove surface rust and apply corrosion-inhibiting primer.

Tighten all loose fasteners and replace missing or damaged fasteners with corrosion-resistant materials.

Clean roof surfaces and prepare seams, ridges, and penetrations for recovery system installation.

B. Flat / Tar (Built-Up) Roofs

The Contractor shall:

Clean roof surfaces of debris, dirt, and loose material.

Inspect existing built-up roofing (BUR) for blisters, cracks, and soft areas.

Perform localized patching or repairs as required to provide a smooth, stable substrate suitable for TPO installation.

Ensure positive drainage prior to installation of recover components.

2.3 TPO RECOVER SYSTEM INSTALLATION

The Contractor shall install a manufacturer-approved TPO recover roofing system over all prepared roof surfaces, including:

Recovery Board / Separation Layer

Install approved recovery board, insulation, or separation layer as required by the TPO manufacturer to provide a continuous substrate.

TPO Membrane Installation

Install a fully adhered TPO membrane over all roof areas.

Heat-weld all seams to form a continuous watertight assembly.

Flashing and Detailing

Install TPO flashing at all penetrations, walls, curbs, parapets, and transitions.

Integrate roof system with existing roof edges, drains, and penetrations.

2.4 GUTTERS AND DOWNSPOUTS

A. Remove and replace all existing gutters and downspouts on buildings included in this contract.

B. New gutters and downspouts shall be corrosion-resistant, properly sized, and securely attached.

C. Downspouts shall discharge away from building foundations and coordinate with existing drainage.

2.5 SOLAR PANEL COORDINATION

A. Some buildings include existing rooftop solar panel systems.

B. The Contractor shall:

Perform roofing work around existing solar panels, or

Temporarily remove and reinstall solar panels as required to complete roofing work.

C. Solar panels shall be:

Protected from damage

Reinstalled in original configuration

Fully operational upon project completion

D. Damage to solar equipment caused by Contractor operations shall be corrected at no additional cost to the Government.

3.0 BUILDINGS INCLUDED

No.	Building					Existing Roof Type		Approx. Roof Area (SF)

1	Smokey Bear Ranger District Office			Metal			4,648

2	Smokey Bear Ranger Small Office			Metal			916

3	Smokey Bear Ranger District Recreation Building		Metal			456

4	Smokey Bear Ranger District Fire Operations Building	Metal			4,125

5	Smokey Bear Ranger District Tanker Bay			Metal			2,400

6	Smokey Bear Ranger House				Flat / Tar (BUR)		1,704

7	Smokey Bear Seasonal Housing				Flat / Tar (BUR)		1,600

Estimated Total Roof Area: ~15,849 SF

4.0 GOVERNMENT-FURNISHED PROPERTY

The Government will provide site access and coordination through the Contracting Officer’s Representative (COR).

No materials or equipment will be furnished by the Government.

5.0 QUALITY CONTROL AND INSPECTION

A. Contractor shall submit product data and installation details prior to mobilization.

B. The COR may inspect work at the following stages:

Completion of roof preparation

Installation of recovery layers

Final membrane, gutter, and solar coordination completion

C. Work not meeting contract requirements shall be corrected at no additional cost to the Government.

6.0 DELIVERABLES

Project schedule

Manufacturer product data

As-built roof and gutter layouts

Final completion documentation

7.0 PERFORMANCE PERIOD

Work shall be completed within 160 calendar days from Notice to Proceed (NTP).

8.0 SAFETY REQUIREMENTS

The Contractor shall comply with all applicable OSHA regulations and Forest Service safety requirements, including fall protection and coordination around occupied facilities.

9.0 DATA RIGHTS

All documentation produced under this contract shall become the property of the U.S. Forest Service.