Repair Cottonwood Cove Main Lift Station Pump Repair

Description:

This project will rebuild old sewer pumps and replace old, worn out or decaying seals and electrical components. The contractor will return functioning pumps after the rebuild.

Scope of work to be completed:

The contractor will clean, disassemble and repair the broken components of 2 (two) sewer lift station pumps. They will replace the mechanical seal on the Grundfos pumps and reassemble. They will test pumps for proper operation. They will install new wiring on pumps if needed.

Contract Requirements:

Contractor shall rebuild NPS owned lift station pumps and return working units and replace mechanical seals and electrical components as needed.

The NPS will drop off and pick up pumps at the selected repair shop. The repair shop must be within 350 driving miles of the NPS warehouse located at: 101Katzenbach drive, Boulder City, NV 89005. NPS will drop off and pick up pumps from the same location of the selected authorized repair shop.

Spec of existing pumps:

Grundfos P/N 99791050 SEV.30.A30.420.2.52H.C.EX.61R.A

submersible sewage pump, cast iron construction with a 3”

horizontal flanged discharge, vortex impeller with 3” solids

handling capability, double mechanical seals with sensor probe.

Pump direct coupled to an air-filled explosion proof submersible

motor with 49’ power cord, rated 42HP 3450 RPM 230/460/3/60.

If during performance of work, it is discovered that if there are additional repairs needed beyond what is indicated in the scope of work, stop and notify the COR and CO immediately and submit a quote for the additional repairs.