Scope of Work (SOW)

Table of Contents

1.0    Project Description / Summary

2.0    Objectives

3.0    Background

4.0    Specific Work / Tasks

5.0   Optional Work / Tasks

6.0    Government Furnished Equipment / Materials / Other

7.0    Special Conditions / Requirements

8.0    Place of Performance

9.0    Performance Period / Schedule Requirements

| Scope of Work |  |
| --- | --- |
| Task Order / Project Title: | Cabin Security and Access Improvements |
| Park / Park Unit: | PAIS |
| Task Order#: | TBD |
| Prepared By: | Greg Smith |
| Date: | 18/Febuary/2026 |

- Project Description / Summary: 
The primary work of this project includes, but is not limited to, the procurement and improve physical security, and construct ramp extension, at the coastal cabin structure near saltwater environment.

- Objectives:

CLIN 001 - Repair Doors and Install Door Reinforcement.

CLIN 002 – Repair Roof Leak

CLIN 003 – Install CAT 5 TDI Hurricane-Resistant Design, Non- Decorative Shutters.

OPTION 001 –. Construct Cabin Ramp Extension

OPTION 002 – Construct Breakaway Sand Control.

- Background: 

Padre Island National Seashore, approximately 50 miles south of Hwy 361 on Park Road 22. Existing building is a coastal wood framed structure near saltwater environment. See attached As-built drawings.

- Specific Work / Tasks:

CLIN 001 - Door Repair and Door Reinforcement.

  - For additional security measures.

    - Adjusting door with Maximum Diagonal Distortion: 1/16 inch (1.6 mm) measured with straight edge, corner to corner. Adjust for smooth and balanced door movement. Rehang door if it does not swing or operate freely.

    - New Corrosion-Resistant Reinforced door handle hardware.

    - Use stainless steel materials for hinges, locks, and door frame screws.

    - Install reinforcement strips on the door frame, particularly around the hinge side, to enhance durability.

    - New Deadbolt and Door Jamb Reinforcement

    - Install a high-quality deadbolt with a long throw bolt for enhanced security.

    - Use a door jamb reinforcement kit to fortify the area around the lock mechanism.

CLIN 002 – Repair Roof Leak

Roof Lea

- Inspection:

  - Perform a thorough visual inspection and water test to locate all leak points.

- Material Replacement:

  - Remove damaged roofing materials and replace with marine-grade, corrosion-resistant like kind shingles.

- Sealing:

  - Apply high-performance waterproof sealant to all joints, seams, and penetrations.

- Fasteners:

  - Use stainless steel or fasteners rated for coastal environments.

- Verification:

  - Conduct a controlled water test post-repair to confirm leak elimination.

4. Special Conditions

- All work must comply with coastal building codes and floodplain requirements.

- Contractor must maintain a clean and safe worksite.

- Schedule work to minimize disruption to park operations.

5. Deliverables

- Completed roof repair meeting specifications.

- Documentation of materials used and repair process.

- Final inspection report signed by park representative.

CLIN 003 – Install CAT 5 TDI Hurricane-Resistant Design, non- decorative shutters.

  - Ten (10) New Shutters must withstand hurricane-force winds. Source Bahama shutters and coordinate with area suppliers and ensure intrusion security. Provide durable, corrosion-resistant Bahama-style hurricane shutters to protect the structure’s windows from hurricane-force winds and potential intrusion during long unoccupied periods. The shutters should be low-maintenance and mechanically operable to suit the lack of electrical power at the site.

    - Bahama shutters comply with the Building Code for High Velocity Hurricane Zones, Texas Department of Insurance (TDI) regulations, and ASTM impact testing approvals. The shutter assembly must have ability to be raised into an open position. The shutter assembly must be lowered into the closed position during a hurricane event. The shutters may be installed as a single panel configuration or as a multi panel configuration. When the storm bar is utilized, the maximum width is limited as specified in the manufacturer’s drawings.

    - Corrosion Resistance Material:  Stainless Steel or Powder-Coated Aluminum, specifically designed for marine environments. Aluminum shutters should be powder-coated to resist corrosion, and resistance against salty moisture exposure. Color approved by Park.

    - Security Features Lockable design: When unoccupied, shutters must be lockable for intrusion resistance. If a factory lock is unavailable, contractor to provide a fabrication solution, such as a padlock mechanism or additional security feature, to secure the shutters effectively.

    - Low Maintenance - Required Non-electric operation with easy robust manual adjustment to open and close. Shutters should require minimal upkeep due to the absence of electrical power on-site.

  - Installation Requirements:

    - Install Bahama shutters directly over each of the ten (10) windows, ensuring correct top-hinge configuration.

    - Secure shutters to the structure using stainless-steel corrosion-resistant screws, bolts, and hardware.

    - All penetration points shall be properly sealed and weatherproofed to meet existing building specifications.

    - Use maximum anchor spacing including top female hinges according to CAT 5 Hurricane-Resistant Design for installation into wood southern pine.

    - Identify maximum shutter span and maximum shutter width according to CAT 5 Hurricane-Resistant Design.

    - Install shutters in accordance with regional hurricane protection codes and any additional structural specifications.

    - Verify that all installed shutters function as intended, providing both storm protection and intrusion security.

  - Hardware and Accessories:

    - Use all corrosion-resistant stainless steel materials, including hardware for hinges, locks, and any required reinforcement.

    - Shutter supports should ensure they are sturdy and withstand both lateral and upward wind loads. Bahama shutters shall have Locking Rod.

    - Custom Security Lock Fabrication Coordinate to add lockable functionality if factory-installed locks are unavailable. Confirm with the CO on custom lock fabrication, using a padlock option if necessary.

    - All screws to be stainless steel series with required strength by the manufacturer and shall comply w/ Texas building code.

    - Bolts to be series stainless steel with required strength by the manufacturer.

    - All aluminum pop rivets to be aluminum alloy with aluminum mandrel.

    - Anchors to wall shall be as per manufacturer requirements.

- Optional Work  /  Tasks:

OPTION 001 – Construct Cabin Ramp Extension.

  - Construct a new ramp extension attached to the existing cabin to improve accessibility. Reference “Attachment 2 Ramp Extension” plan. See “att1 PAIS Cabin 1.25.23 - AS BUILTS”.

    - Includes two new landings, two new ramp runs, and the integration of Government-Furnished Equipment (GFE) aluminum ramp to the existing grade.

    - The design and construction shall adhere to specifications outlined in the as-built drawings and comply with the details provided below.

  - Structural Components

    - Joists and Decking: Joists: Minimum 2x8 pressure-treated lumber, spaced 16 inches on center (o/c). Deck Boards: 2x6 pressure-treated decking boards, securely fastened to joists.

    - Handrails. Posts: 4x4 pressure-treated posts lag-screwed to the rim joists with a minimum of (2) 5-inch A307 lag screws (1/4-inch diameter minimum) or equivalent structural screws. Posts must extend at least 4 inches onto the rim joists. Handrails consist of 2x4 top, middle, and bottom rails. Top cap to be installed at a minimum height of 36 inches above the deck boards. 4x4 columns must be positioned to prevent rail deflection. Install handrails as detailed in as-built drawings. Not explicitly shown in the new ramp extension plans.

    - Ramp Dimensions and Slope. Width: Ramp and stair tread shall be 46 to 48 inches wide. Slope: Ramp slope shall be 1.5 inches per foot. Landing: Landing shall terminate approximately 3 feet above grade.

    - Pressure Blocking. Pressure blocking must be installed where stringers hang on ledgers or rim joists/beams. Secure blocks with a minimum of (10) 3-inch heavy-duty (HD) nails per connection type.

    - Beams and Posts. Beams: 2x12 pressure-treated beams. Attach beams to 6x6 pressure-treated posts using one of the following methods:

      - (2) A307 5/8-inch diameter x 4-inch lag screws.

      - (3) 5/16-inch x 4-inch GRK structural screws rated for treated wood.

  - Construction Notes: All materials used shall be pressure-treated and rated for outdoor exposure. Work shall comply with requirements for slope and accessibility where applicable. Contractor must confirm final dimensions, alignment, and grade with park representatives before installation. All fasteners shall be corrosion-resistant and suitable for treated wood. Ensure all components are level, plumb, and securely fastened.

  - Responsibility of the Contractor: Provide all labor, tools, and materials (excluding GFE aluminum ramp) necessary to complete the work. Coordinate installation of the GFE aluminum ramp with park representatives. Ensure proper integration of the ramp extension with the existing grade and structure. Maintain a clean and safe worksite throughout construction.

OPTION 002 – Construct Breakaway Sand Control.

  - Construct a breakaway treated lumber (2x10s) boards maximum two boards high, around three sides of the ground perimeter of the existing raised coastal cabin structure. The sand control will provide sand shielding for equipment and vehicles stored underneath the cabin. The cabin is raised on pilings, and vehicles routinely park beneath it. The sand control will enhance protection against sand and debris infiltration while maintaining airflow and accessibility for maintenance.

  - Materials: Treated Lumber: 2x10 pressure-treated wood rated for ground contact to ensure durability against coastal environmental conditions. Fasteners: Stainless steel resistant to corrosion in high-salinity environments.

  - Installation Requirements: Measure and mark the ground perimeter around the raised cabin to determine the precise dimensions of the sand control for three sides of the structure. Ensure the sand control can breakaway and aligns uniformly with the cabin’s structural supports and clearances.

    - Breakaway enclosures are designed to fail under base flood conditions without jeopardizing the elevated building any below the enclosure must be breakaway. Breakaway enclosures are permitted but must be equipped with flood openings.

    - Clear the ground of debris and level uneven areas along the perimeter to provide a stable base for the sand control. Attach each panel to the pilings or framing of the raised cabin using corrosion-resistant stainless-steel fasteners.

    - Sand Sealing: Ensure the lower edge of the sand control fits snugly against the ground. If necessary, trench to minimize sand intrusion.

- Government Furnished Equipment / Materials / Other:

One Government Furnished Equipment (GFE) Aluminum Ramp.

- Special Conditions / Requirements:

  - Inspection and Acceptance

    - The completed ramp extension will be inspected by park representatives to ensure compliance with the scope of work and as-built drawings. Any deficiencies must be addressed and corrected by the contractor before final acceptance.

    - Perform operational checks to ensure that shutters are easy to open and close manually and that locks function securely.

    - Conduct a final inspection with park representatives to confirm adherence to all scope specifications.

CLIN 001 - Repair Doors and Install Door Reinforcement.

CLIN 002 – Repair Roof Leak

CLIN 003 – Install CAT 5 TDI Hurricane-Resistant Design, Non- Decorative Shutters.

OPTION 001 –. Construct Cabin Ramp Extension

OPTION 002 – Construct Breakaway Sand Control.

  - Documentation and Training

    - Provide an operation manual for shutter maintenance. Offer a brief training on shutter use and security locking mechanisms for park personnel.

- DELIVERABLES

    - Approved CAT5 TDI Hurricane Bahama shutters on all designated windows. Provide Manufacturers drawings stamped by engineer.

    - Documentation of the locking mechanism if custom fabricated.

    - Operation and maintenance manual for shutters and any reinforced doors with part numbers.

    - Approved Coordination Drawing Details: of ramp extension, including connections, fasteners and connectors, materials used, and depth of 6x6 posts.  Deviations from original drawings.

    - As-built Drawings of constructed ramp extension.

- Place Of Performance:

  - Padre Island National Seashore, approximately 50 miles south of Hwy 361 on Park Road 22. Coastal wood framed structure near saltwater environment.

- Performance Period / Schedule Requirements:

  - All on-site work must be completed before 01 JUNE 2025.

  - Some work can be completed during weekends with 48-hour prior approval.