Salient characteristics means those particular characteristics that specifically describe the essential physical and functional features of the material or service required.  List all the minimum specification, characteristics, capabilities, etc. required for this supply.

PSC:  3695                     Miscellaneous Special Industry Machinery

NAICS: 333248             All Other Industrial Machinery Manufacturing

The Fiber Draw Tower, with the appropriate choice of options, has the following specs:

| General Description of the requirement |  |
| --- | --- |
| Silica fiber optic draw tower |  |
| Resistive heated furnace |  |
| Preform feed mechanism |  |
| Startup belt tractor |  |
| Four independent channels of preform pressurization/vacuum |  |
| Two independent fiber coating systems with 365nm LED UV curing capability |  |
| Two independent fiber coating measurement systems |  |
| Take-up capstan and fiber spooling system |  |
| Automatic closed loop doc for all systems |  |
| Software access that allows future in house hardware additions |  |
| Technical Specifications silica fiber drawing |  |  |
| System…………………………………………………………………………. | Fully turnkey operation, all necessary hardware/software and ancillary equipment, must fit within 1.2m wide x 1.2m deep x 9m tall footprint |  |
| Furnace…………………………………………………………………………. | Resistive, water cooled 2300C max |  |
| Preform Diameter…………………………………………………………. | 10 mm to 50 mm (12-30mm typical) |  |
| Preform Feeding Length………………………………………………. | 1.5 m minimum |  |
| Process Speed………………………………………………………………. | 1 to 200 m/min (1-50 m/min typical) |  |
| Fiber Diameter Measurement………………………………………. | data logging |  |
| Fiber Diameters……………………………………………………………. | 80 µm to 1000 µm (+/-0.1um) |  |
| Preform pressurization…………………………………………………. | 4 independent channels +500mBar inert gas / -900mBar vacuum, (+/-)0.1mBar |  |
| Fiber types……………………………………………………………………. | Step Index MM, Graded Index MM, Single Mode, Microstructured PCF, microstructured hollow core |  |
| Fiber Tapering w/ simultaneous pressure variation………. | Speed and pressure programming, for simultaneous tapering and PCF hole size variation. |  |
| Fiber draw tension monitoring……………………………………… | 0-500gm minimum |  |
| Technical Specifications for silica cane and tubes |  |  |
| Belt / Caterpillar Capstan……………………………………………… | Adjustable grip pressure for thin wall tubing capability |  |
| Preform diameters…………………………………………………….... | 10 mm to 50 mm |  |
| Preform length……………………………………………………………. | 1.5 m minimum |  |
| Cane / Tube Diameters…………………………………………………. | up to 5mm |  |
| Process speed………………………………………………………………. | 0.1 to 10 m/min |  |
| Technical Specifications for coating |  |  |
| Two independent coating systems………………………………. | Heated coating reservoir & die base, pressurization control, coating bubble stripper, standard coating dies (matched sets heads 1& 2, 3 fiber size ranges 125um, 250um, and 400um fiber production) |  |
| Coating Diameter measurement…………………………………… | 2pcs. (1 ea. coating head), data logging |  |
| Coating curing………………………………………………………………. | 2 pcs. LED UV 365nm 12W/cm, air cooled, (1 ea. Coating head) |  |
| Optical sight window……………………………………………………. | Optical sight window for coating meniscus observation |  |
| Data Logging………………………………………………………………… | Minimum, preform feed speed, fiber draw speed, fiber length fiber diameter, furnace temperature, fiber tension, preform pressure/vacuum, concentricity, preform length used |  |
| Software access………………………………………………. | Software access license with non-disclosure agreement. |  |
|  | NRL may at times need to add functionality which may fall under ITAR or other foreign disclosure restrictions. It is therefore necessary for NRL to gain access to software in order to make certain software modifications or hardware additions. |  |

All specifications are subject to change pending draw tower market survey, study of fiber market trends and engineering review by NRL personnel.