1 Model 8.0 Heavy Duty Tiger Tug for LUH-72A / EC145

Center notch in the tug’s top frame

to accommodate the MEPS system

Tug Configuration:

All tugs built for Army Aviation Support Facilities have a notch in the center top frame to provide clearance for the helicopter’s MEPS system.

Model 8.0 Heavy Duty Overview:

Compared to the standard Model 8.0 tug, the Model 8.0 Heavy Duty tug incorporates several design changes for high frequency tug usage, longer travel distances, and/or increased slopes between the hangar and tarmac.

The Model 8.0 Heavy Duty tug utilizes a 36V system voltage instead of the standard 24V system voltage, which results in lower resistive heating of the tug’s wiring to reduce the potential for overheated wiring.

The Model 8.0 Heavy Duty tug also utilizes a more powerful electric motor and corresponding transaxle and dedicated motor controller.  The photo below shows an example of the electric motor and transaxle at the r