Table of Contents

1. OVERVIEW	1

2. CLIN 0001: Dynamic Waterjet Cutting Head Assembly	1

3. CLIN 0002: Control and Programming Software	2

4. COMMON SAFETY AND DOCUMENTATION REQUIREMENTS	2

5. INSTALLATION AND TRAINING REQUIREMENTS	3

6. DELIVERY ADDRESS	3

7. TECHNICAL POC	3

### 1. OVERVIEW

This requirement consists of a complete upgrade kit to modernize an existing waterjet cutting machine with a dynamic cutting head for taper control and bevel cutting.

| CLIN | Type | Qty |
| --- | --- | --- |
| CLIN 0001 | Dynamic Waterjet Cutting Head Assembly | 1 ea |
| CLIN 0002 | Control and Programming Software Suite | 1 ea |

### 2. CLIN 0001: Dynamic Waterjet Cutting Head Assembly

TECHNICAL REQUIREMENTS:

- System Functionality:

  - The system must be a fully articulating, dynamic waterjet cutting head (wrist assembly).

  - It must be capable of active taper control to produce straight, taper-free edges on finished parts.

  - The assembly must enable the cutting of beveled edges.

  - Operating Pressure: The system must be rated for a minimum operating pressure of 60,000 PSI (60 KSI) or approximately 4,100 bar.

- Mechanical Components:

  - The delivery must include all necessary mechanical parts for a complete upgrade and installation. This includes, but is not limited to:

    - The complete wrist assembly.

    - A spray and splash containment system (Flow Shields or equivalent).

    - All required high-pressure tubing, fasteners (screws, washers), and mounting hardware.

  - The system must be designed for compatibility with M200-series waterjet systems or equivalent machine frames.

### 3. CLIN 0002: Control and Programming Software

TECHNICAL REQUIREMENTS:

- Software Suite:

  - An integrated CAD/CAM software suite for creating, importing, and simulating 2D and 3D cutting paths must be provided.

  - The software must fully support the functionality of the dynamic cutting head, including automatic calculation of tool paths for taper compensation and bevel cutting.

  - License: The software must be provided with a perpetual, non-expiring license for at least one user station.

  - The delivery must include the means to download and install the software.

### 4. COMMON SAFETY AND DOCUMENTATION REQUIREMENTS

- The system and all components must be brand new and CE compliant.

- An EC Declaration of Conformity according to the Machinery Directive is required.

- Warranty: The warranty for the system, including all hardware and software components, shall be a minimum of 12 months from the date of final acceptance.

- Documentation: 2 complete sets of operation manuals and maintenance manuals must be provided. All documentation must be available in both English and German languages.

### 5. INSTALLATION AND TRAINING REQUIREMENTS

- Installation: The supplier is responsible for the complete installation and commissioning of all hardware and software components, rendering the system fully operational.

- Training:

  - The supplier must provide comprehensive on-site training for operators and maintenance personnel.

  - Software training must cover all aspects of the new CAD/CAM suite, from basic operation to advanced pathing for dynamic cutting.

  - The software training must have a duration of at least 4 days.

### 6. DELIVERY ADDRESS

Kaiserslautern Army Depot
TLSC-E&A / Maintenance Activity Kaiserslautern
Ludwigshafener Strasse 31, Bldg 2233
67657 Kaiserslautern
Germany

### 7. TECHNICAL POC

Markus Feth
E-Mail: markus.feth.ln@army.mil
CIV: (0611) 143 547-0011